Internet of Things Fundamentals

Subject Project

BS AI 6th Semester SP-25 (AIE-3079)

Date: 26-06-2025

Project Title: Smart Environment Monitor: IoT Smoke Detector & Air Index Calculator

Group Name/no.: AI_ZONE / 10

Team Members:

| Members | Registration no | Name | Signature |
| --- | --- | --- | --- |
| Member-1 (Leader) | 22-NTU-CS-1336 | ABDUL RAFAY |  |
| Member-2 | 22-NTU-CS-1355 | M. FAISAL |  |
| Member-3 | 22-NTU-CS-1375 | SUFHAN SIDDIQUE |  |
| Member-4 |  |  |  |

| Contributions in % of each Team Members for each component |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Member-1 | Member-2 | Member-3 | Member-4 |
| Distribution Components |  | RAFAY | FAISAL | SUFHAN | Name |
| Coding | ESP32-coding | 40% | 30% | 30% |  |
|  | Python Coding | 30% | 40% | 30% |  |
| UI Design |  | 30% | 40% | 30% |  |
| Database |  | 30% | 40% | 30% |  |
| Cloud Integration |  | 40% | 30% | 30% |  |
| IoT Gateway |  | 40% | 30% | 30% |  |
| Edge Processing |  | 30% | 40% | 30% |  |
| Documentation |  | 30% | 30% | 40% |  |
| Presentation Design |  | 30% | 30% | 40% |  |
| Replace for other contribution |  |  |  |  |  |
| Replace for other contribution |  |  |  |  |  |
| Replace for other contribution |  |  |  |  |  |
| Replace for other contribution |  |  |  |  |  |

To be filled by the evaluator

Team-Based Evaluation (60 Marks)

| Criteria | Obtained Marks | Out of |
| --- | --- | --- |
| System Design & Architecture |  | 10 |
| Hardware Integration & Circuit Setup |  | 10 |
| IoT Gateway and Cloud Communication |  | 10 |
| Working Prototype Demonstration |  | 10 |
| Performance & Reliability Testing |  | 10 |
| Presentation |  | 10 |
| Total (Team-Based) |  | 60 |

Individual-Based Evaluation (40 Marks per Member)

|  | Member 1 | Member 2 | Member 3 | Member 4 |
| --- | --- | --- | --- | --- |
| Criteria |  |  |  |  |
| Understanding of the Project & Role | /10 | /10 | /10 | /10 |
| Code Contribution and Explanation | /10 | /10 | /10 | /10 |
| Q/A VIVA | /10 | /10 | /10 | /10 |
| Documentation/Reporting & Communication | /10 | /10 | /10 | /10 |
| Total (Individual-Based) | /40 | /40 | /40 | /40 |
| Total Overall (60+40) | /100 | /100 | /100 | /100 |
| Weightage Lab Grade (50) |  |  |  |  |

1. Abstract / Executive Summary

This project presents a smart IoT-based Smoke and Air Quality Monitoring System using an ESP32 microcontroller. It integrates a smoke sensor (MQ-2) and an air index sensor (MQ-135) to detect harmful gases and pollution in real time. The system alerts users through LED indicators and sends live data to the Blynk Cloud, where it is visualized via a mobile application. This solution promotes environmental monitoring and safety, especially in indoor settings prone to fire or poor air quality.

2. Table of Contents

Abstract / Executive Summary

Table of Contents

Introduction

Literature Review

Methodology / System Design

Implementation

Results & Discussion

Testing & Validation

Conclusion & Future Work

References

Links

3. Introduction

Background & Motivation

Air pollution and fire hazards pose significant health and safety threats. Many indoor spaces lack real-time monitoring tools for smoke and air quality.

Problem Statement

There is a lack of affordable, real-time air quality and smoke detection solutions that also provide remote monitoring capabilities.

Project Goals

Detect harmful smoke and poor air quality

Alert locally with LEDs

Send live data to the cloud via ESP32

View data on the Blynk mobile app

4. Literature Review

Relevant Concepts:

ESP32: Wi-Fi enabled microcontroller ideal for IoT

MQ-2 & MQ-135: Sensors used to detect smoke and gas pollutants

Similar Projects:

Fire detection systems with Arduino

Indoor AQI monitors with Blynk

IoT-based smart homes using ESP modules

5. Methodology / System Design

5.1 Hardware Components:

ESP32 microcontroller

MQ-2 smoke sensor

MQ-135 air quality sensor

LEDs (Red/Green)

220-ohm resistors

Jumper wires & Breadboard

5V USB supply

buzzer

Circuit Diagram:

MQ-2 and MQ-135 connected to ADC pins (A0, A1)

LEDs connected to GPIOs via resistors

5.2 Software Design:

Libraries/Tools:

MicroPython via Thonny

urequests (for Blynk HTTP API)

Blynk Cloud and Mobile App

Flowchart/System Logic:

Connect to Wi-Fi

Read sensor values

Turn ON LED if unsafe

Send data to Blynk

Loop

Pseudocode:

connect_to_wifi()

while True:

smoke = read_mq2()

air = read_mq135()

if smoke > threshold or air > threshold:

turn_on_led()

send_to_blynk(smoke, air)

sleep(5)

6. Implementation

Arduino Code:

// WiFi and MQTT details

const char* ssid = "iPhone 13 Pro";

const char* password = "11223344";

const char* mqtt_server = "172.20.10.8";

const int mqtt_port = 1883;

const char* topic = "esp32/air_quality";

void reconnect_mqtt() {

while (!client.connected()) {

Serial.print("Connecting to MQTT...");

if (client.connect("ESP32_AirQuality")) {

Serial.println("connected");

} else {

Serial.print("failed, rc=");

Serial.print(client.state());

delay(2000);

}

setup_wifi();

client.setServer(mqtt_server, mqtt_port);

}

void loop() {

if (!client.connected()) {

reconnect_mqtt();

}

client.loop();

int mq2Value = analogRead(MQ2_PIN);

int mq135Value = analogRead(MQ135_PIN);

if (mq2Value < 800) {

digitalWrite(GREEN_LED, HIGH);

quality = "GOOD";

} else if (mq2Value < 1800) {

digitalWrite(BLUE_LED, HIGH);

quality = "MODERATE";

} else {

digitalWrite(RED_LED, HIGH);

ledcWrite(buzzerChannel, 128);  // Half duty (50%)

quality = "BAD";

}

// MQTT Publish

char payload[150];

snprintf(payload, sizeof(payload),

"{\"mq2\": %d, \"mq135\": %d, \"air_quality\": \"%s\"}",

mq2Value, mq135Value, quality.c_str());

client.publish(topic, payload);

Serial.print("Published: ");

Serial.println(payload);

delay(5000);

Python Script:

# MQTT broker config

broker = '172.20.10.8'  # HiveMQ IP

port = 1883

topic = "esp32/air_quality"

client_id = 'mqtt_influx_bridge'

# InfluxDB config

influx_url = "http://localhost:8086"

token = "R5pxqAZrrxiCgsCexGALmwG2PeVaGyqNsMpPAfSy89ufz6z8QuvZCKvz3qlHKaGGILdxkqvf17FkIIlvY9hetw=="

org = "ntu"

bucket = "air_quality_monitor"

def connect_mqtt():

def on_connect(client, userdata, flags, rc):

if rc == 0:

print("Connected to MQTT Broker!")

else:

print(f"Failed to connect, return code {rc}")

client = mqtt_client.Client(client_id=client_id)

client.on_connect = on_connect

client.connect(broker, port)

return client

# Write to InfluxDB

write_api.write(bucket=bucket, org=org, record=point)

print("Written to InfluxDB")

except Exception as e:

print(f"Error processing message: {e}")

def run():

client = connect_mqtt()

client.subscribe(topic)

client.on_message = on_message

client.loop_forever()

Challenges:

Sensor calibration was tricky

urequests sometimes failed due to weak Wi-Fi

Debouncing LED flickers

ESP32 faces challenges in sensor accuracy, data handling, and integrating with other systems

5v Power supply to sensors was tricky

7. Results & Discussion

Outputs:

Sensor readings in serial monitor

Data updates in Blynk app (Gauge, Label widgets)

Performance:

Real-time response ~1s delay

Blynk update stable under 2.4GHz Wi-Fi

MQ sensors show reliable trend changes

Comparison:

Smoke alerts matched real ignition sources

AQI levels increased with lighter spray/smoke test

8. Testing & Validation / Limitations

Test Cases:

Test with candle smoke: LED turns on, Blynk updates

Test with air spray: AQI spike observed

Disconnect Wi-Fi: ESP fails to send, handles exception

Limitations:

No GSM/Offline support

Blynk free plan has data rate limits

Sensors need calibration for real AQI

9. Conclusion & Future Work

Key Takeaways:

Low-cost, real-time detection of air and smoke

Simple Blynk-based dashboard for remote access

MicroPython is suitable for rapid IoT development

Future Improvements:

Add SMS alert

Integrate temperature/humidity sensors

Use AI/ML for pattern detection (fire prediction)

10. References

MQ-2 Datasheet

MQ-135 Info

https://www.youtube.com/watch?v=4cv-6hddJdY

https://www.youtube.com/watch?v=kJ0vG7ZFZ5I

ESP32 MicroPython Guide

https://docs.blynk.io/en/blynk.cloud/blynk-cloud-api

11. Links:

GitHUB Link:

https://github.com/Cyberpunk131/IOT_Lab_work.git

Google Drive Link:

https://drive.google.com/drive/folders/1A54TaqGEvEFDTxvwQCCW43FCw4xULoCz